# Task 6

Write a test to change behaviour of a locked pmp region using smepmp extension in spike (Smepmp task)

a. Pre-requisites

    - Read RISCV privileged architecture spec for PMP

    - Read about Smepmp extension

b. Write an assembly test.

    - Start your test in M mode. Configure a locked pmp region for read, write and execute permissions.

    - What two things happen when you configure a locked pmp region??

    - Now, try to change permissions of that region to only read from read,write and execute. How it can be achieved using smepmp extension

## ➢ Solution:

Github Link:

https://github.com/BilalAli10x/Riscv_Arch_Test/tree/main/BilalAli_Task6

Test Description

This test verifies:

- Behavior of a locked PMP region (L=1).

- That locked PMP entries:

  - Become immutable.

  - Apply to M-mode.

- How Smepmp (mseccfg.RLB) allows modification of locked PMP entries.

- Runtime permission change from RWX -> Read-only.

- Behavior difference:

  - With SMEPMP disabled

  - With SMEPMP enabled

The test runs on Spike and validates behavior using CSR readback and trap detection.

Q): What Two Things Happen When PMP Region is Locked?

From Privileged Spec:

When L = 1 for a PMP entry:

1) Entry Becomes Immutable

- Writes to:

  - pmpcfgX

  - pmpaddrX

- Are ignored.

- The entry cannot be modified until reset.

2) Entry Applies to M-mode

Normally:

- PMP does NOT restrict M-mode.

But when L = 1:

- The region also applies to Machine mode.

- M-mode accesses are checked against that PMP entry.

This is explicitly defined in:
Privileged Spec – PMP Section 3.7.1 (Page no 60)

Why Locked PMP Cannot Be Modified Normally

- Without Smepmp:

csrw pmpcfg0, new_value

- If entry has L=1:

-> Write is silently ignored.

- Readback:

csrr t1, pmpcfg0

Will show old value.

How Smepmp Solves This

From Smepmp extension:

- The mseccfg CSR introduces:

-> RLB (Rule Locking Bypass) Bit

- If:

mseccfg.RLB = 1

- Then:

Locked PMP entries may be modified.

Lock protection is bypassed.

- BUT -> Important rule from spec:

If any PMP entry has L=1 while RLB=0, then RLB becomes read-only 0.

- So:
 -> RLB must be set before locking behavior permanently blocks it.

Implementation Overview

Step 1: Configure PMP Regions

Two TOR regions configured:

| Region | Mode | Permissions | Lock |
| --- | --- | --- | --- |
| 0 | TOR | RWX | 0 |
| 1 | TOR | RWX | 1 |

Configuration written:

- pmpcfg0 = 0x00008F0F

- Region 1:

L=1, A=TOR, X=1, W=1, R=1

Step 2: Test Locked Region (Initial RWX)

Operations performed in M-mode:

- Store -> Success

- Load -> Success

- Execute -> Success

Because region is RWX.

Step 3: Attempt to Change Locked Region to Read-Only

New configuration attempted:

pmpcfg0 = 0x0000890F

Region 1 becomes:

Now behavior depends on SMEPMP.

Case 1 – WITHOUT SMEPMP

- (#define SMEPMP commented)

- (#define NO_SMEPMP uncommented)

Expected Behavior:

- Write to locked PMP entry is ignored.

- Readback shows old configuration.

- RWX permissions remain active.

- No traps occur.

CSR Check in Code

- csrr t1, pmpcfg0

- beq t1, t4, fail_cfg_change

If configuration changed -> FAIL
Since it should NOT change.

Access Results

| Operation | Result |
| --- | --- |
| Write | Success |
| Read | Success |
| Execute | Success |

Final Output:

PASS (gp = 1)

Spike Log Snapshot:

Case 2 – WITH SMEPMP (#define SMEPMP enabled)

- (#define SMEPMP commented)

- (#define NO_SMEPMP uncommented)

Before configuring PMP:

- li t0, (1 << 2)

- csrw mseccfg, t0

This sets:

- mseccfg.RLB = 1

What RLB Does

Rule Lock Bypass allows:

- Modification of locked PMP entries.

Expected Behavior

- Configuration change succeeds.

- Readback matches new value.

- Region becomes Read-only.

- Write and Execute cause traps.

Access Results

| Operation | Result |
| --- | --- |
| Write | Store Access Fault |
| Read | Success |
| Execute | Instruction Access Fault |

Trap handler confirms:

- mcause = 1 -> Instruction fault

- mcause = 7 -> Store fault

Final Output

PASS (gp = 1)

Spike Log Snapshot:

Reference to Specification

RISC-V Privileged Architecture Specification

- Section 3.1.15 - mcause

  - Instruction page fault = 12

  - Load page fault = 13

  - Store/AMO page fault = 15

- Section 3.7:PMP Section - Physical Memory Protection CSRs

- Section 3.7.1:PMP Section - Address matching and L-bit behavior

- SMEPMP Spec Chapter 2: Proposal behavior of bit 2