# Task 7

Write a test to verify smepmp extension behaviour (Smepmp task)

a. Pre-requisites

    - Read RISCV privileged architecture spec for PMP

    - Read about Smepmp extension

b. Write an assembly test.

    - Start your test in M mode.

    - Set mseccfg.MML

    - Add a rule with executable privileges that either is M-mode-only or a locked Shared-Region?? what happens and why

    - How can you solve the above problem

Github Link:

https://github.com/BilalAli10x/Riscv_Arch_Test/tree/main/BilalAli_Task7

Test Description

This test verifies the behavior of the Smepmp (Supervisor Mode Execution Prevention PMP) extension, specifically:

- How enabling mseccfg.MML before configuring PMP affects execution.

- How configuring PMP before enabling MML allows proper execution.

The goal is to demonstrate the order-dependency: MML changes how PMP regions interpret privileges, even in M-mode.

Q): Add a rule with executable privileges that either is M-mode-only or a locked Shared-Region? what happens and why?

Observed Behavior:

- Setting MML before configuring the PMP region triggers an instruction access fault:

Reason:

When mseccfg.MML (Machine Mode Lockdown) is set to 1, the rules for memory access change instantly:

- Setting MML automatically enables the behavior of MMWP (Machine Mode Whitelist Policy).

- MML sets Machine Mode Lockdown, making PMP rules strictly enforceable.

- If PMP region is configured after MML is set, M mode loses its default “allow-all” permission. It can now only execute or access memory regions that are explicitly defined in PMP rule.

- At address 0x80000010, enabled MML. And at address 0x80000014, the processor tries to fetch the next instruction (la t0, _data_end). Since nott configured pmpcfg0 yet, there is no "whitelist" entry allowing execution at 0x80000014. The hardware sees a fetch to an unprotected region and triggers the trap_instruction_access_fault.

Q):How can you solve the above problem

## ➢ Solution:

- Step 1: Configure PMP regions first

la t0, _data_end

srli t0, t0, 2

csrw pmpaddr0, t0

li t4, 0x8C     # TOR + Execute-only

csrw pmpcfg0, t4

- Step 2: Enable Machine Mode Lockdown (MML) afterwards

li t0, 1

csrw mseccfg, t0

- Effect:

  - Execution in M-mode works correctly.

  - No instruction access fault occurs.

  - PMP rules are already in place before enabling MML, ensuring proper enforcement.

Results:

- MML changes how PMP rules are enforced by making them strictly applied, even in M-mode.

- If MML is set before configuring PMP, any newly added PMP region may immediately restrict execution access.

- This leads to an instruction access fault because the processor checks the PMP configuration under lockdown rules.

- By configuring the PMP region before enabling MML, the intended permissions are properly established.

- When MML is later enabled, it applies on the already-correct configuration rather than applying restrictions to an incomplete setup.

- This confirms that SMEPMP behavior is order-dependent, and correct initialization sequence is essential to avoid execution faults.

Reference to Specification

RISC-V Privileged Architecture Specification

- Section 3.7:PMP Section - Physical Memory Protection CSRs

- Section 3.7.1:PMP Section - Address matching and L-bit behavior

SMEPMP Specification

- Chapter 2: Proposal behavior of bit 1